# Project 1: Stack Detective – “Where Did My Variable Go?”

## Objective

The objective of this experiment is to understand how local variables are stored in memory and how stack memory behaves during nested function calls. The project helps visualize stack growth and understand memory organization in embedded systems.

## Learning Objectives

- Understand where variables are stored in memory.

- Observe stack growth during function calls.

- Understand function call behavior.

- Learn how local variables are automatically allocated and released.

- Understand the role of stack memory in embedded systems.

## Hardware and Software Used

### Hardware

- ESP32 Development Board / Arduino Uno

- USB Cable

- Computer

### Software

- Arduino IDE

- Serial Monitor

## Theory

Memory in embedded systems is divided into different regions such as:

- Program Memory (Flash)

- Global/Data Memory

- Stack Memory

- Heap Memory

Local variables declared inside functions are stored in the stack memory. Whenever a function is called, a new stack frame is created. The local variables of that function are placed inside the stack frame.

When the function returns, the stack frame is automatically removed and the memory becomes available again.

The stack follows the Last In First Out (LIFO) principle.

## Program Description

Three nested functions were created:

- level1()

- level2()

- level3()

Each function contains a local integer variable.

The address of each variable is printed using the Serial Monitor. By comparing the addresses, the growth direction of the stack can be observed.

## Program Code

void level3()
{
    int c = 30;
    Serial.printf("Level3 variable address: %p\n", &c);
}

void level2()
{
    int b = 20;
    Serial.printf("Level2 variable address: %p\n", &b);
    level3();
}

void level1()
{
    int a = 10;
    Serial.printf("Level1 variable address: %p\n", &a);
    level2();
}

void setup()
{
    Serial.begin(115200);
    level1();
}

void loop(){}

## Experimental Procedure

- Connect the ESP32/Arduino board to the computer.

- Upload the program using Arduino IDE.

- Open Serial Monitor.

- Observe the printed memory addresses.

- Record the addresses of variables a, b, and c.

- Compare the addresses to identify stack growth direction.

## Observation Table

| Variable | Address |
| --- | --- |
| a | __________ |
| b | __________ |
| c | __________ |

### Sample Observation

| Variable | Address |
| --- | --- |
| a | 0x3FFB1A8C |
| b | 0x3FFB1A6C |
| c | 0x3FFB1A4C |

## Analysis

The addresses of variables changed with each function call.

It was observed that:

- Variable a was allocated when level1() was called.

- Variable b was allocated when level2() was called.

- Variable c was allocated when level3() was called.

The address values decreased as the function call depth increased. This indicates that the stack grows downward in memory.

Each function call created a new stack frame containing its local variables and return information.

When level3() completed execution, its stack frame was removed automatically. The same happened for level2() and level1().

## Results

The experiment successfully demonstrated stack memory allocation during nested function calls.

The memory addresses confirmed that:

- Local variables are stored on the stack.

- New stack frames are created for every function call.

- Stack memory grows downward.

- Stack memory is automatically released when functions return.

## Conclusion

This experiment provided a practical understanding of stack memory organization in embedded systems. By printing variable addresses from nested functions, the growth direction of the stack was visualized. The experiment demonstrated that local variables are automatically allocated on the stack and removed when the function exits.

Understanding stack behavior is essential in embedded systems because excessive stack usage can lead to stack overflow and system crashes.

## Learning Outcome

From this experiment, I learned:

- Local variables are stored in stack memory.

- Every function call creates a new stack frame.

- Stack memory follows the Last In First Out (LIFO) principle.

- Stack memory grows downward in most embedded systems.

- Memory allocated on the stack is automatically released when a function returns.

- Understanding stack behavior helps prevent stack overflow issues in embedded applications.