# Day 2 Work Report – Pygmy Development Environment Setup

Name: Prajwal
Date: 09 June 2026

## Project

Pygmy FPGA Development Environment Setup and Toolchain Compilation

## Objective

To set up the Pygmy development environment on Termux, configure the required FPGA toolchain, and compile QuickLogic Yosys for the target platform.

## Tasks Performed

### 1. Repository Setup

- Cloned the Pygmy development repository from GitHub.

- Verified repository structure and downloaded all required submodules using recursive cloning.

- Confirmed successful installation of:

  - Pygmy SDK

  - QuickLogic Yosys

  - QuickLogic FASM

  - Yosys SymbiFlow Plugins

  - VTR (Verilog-To-Routing)

### 2. SymbiFlow Toolchain Setup

- Created SymbiFlow installation directory.

- Extracted the architecture package (arch.tar.gz).

- Configured environment variables and installation paths.

- Verified successful extraction of QuickLogic architecture files.

### 3. Build Environment Verification

- Installed required development tools and dependencies in Termux.

- Investigated package compatibility issues.

- Replaced deprecated packages with compatible alternatives where necessary.

### 4. Yosys Compilation

- Started compilation of QuickLogic Yosys.

- Successfully configured the GCC build environment.

- Identified multiple platform-specific compilation issues during the build process.

## Issues Encountered and Resolution

### Issue 1: Missing Archive File

Problem:

- Installation failed because arch.tar.gz was not found.

Resolution:

- Verified file location.

- Downloaded and extracted the correct archive successfully.

### Issue 2: Missing Python Development Package

Problem:

- The package “python-dev” was unavailable.

Resolution:

- Replaced obsolete package references with Python 3 development packages.

### Issue 3: Missing Tcl Dependencies

Problem:

- Build process reported missing Tcl-related components.

Resolution:

- Installed and verified Tcl development packages.

### Issue 4: Missing spawn.h Header

Problem:

- Yosys compilation failed with:

  - spawn.h file not found

Investigation:

- Analyzed source files.

- Traced the issue to the RPC frontend module.

- Confirmed that the repository and submodules were complete.

- Determined that the issue was related to Termux platform compatibility.

Resolution:

- Located RPC frontend build configuration.

- Disabled rpc_frontend.o from compilation.

- Successfully resumed the build process.

### Issue 5: Missing S_IREAD and S_IWRITE Definitions

Problem:

- ABC compilation failed due to undefined file permission macros.

Resolution:

- Investigated source code.

- Replaced legacy permission definitions with POSIX-compatible alternatives suitable for Termux.

### Issue 6: ABC Local Modification Detection

Problem:

- Build system detected modifications in ABC source files and halted compilation.

Resolution:

- Investigated build configuration and prepared the build system for modified source handling.

## Technical Skills Applied

- Linux Command Line

- Git and Submodule Management

- Build System Debugging

- Source Code Analysis

- Embedded Development Environment Setup

- Dependency Resolution

- FPGA Toolchain Configuration

- Cross-Platform Compatibility Troubleshooting

## Current Status

- Repository setup completed.

- Toolchain extraction completed.

- Yosys compilation significantly progressed.

- Multiple platform compatibility issues identified and resolved.

- Build process is continuing with further validation and debugging.

## Next Steps

- Complete QuickLogic Yosys compilation.

- Build Yosys SymbiFlow Plugins.

- Compile VTR components.

- Verify FPGA synthesis flow.

- Execute sample FPGA design build and validation.

## Summary

Successfully established the Pygmy FPGA development environment, configured the required toolchain, and advanced the QuickLogic Yosys compilation process. Several platform-specific compatibility issues were analyzed and resolved through systematic debugging, source code investigation, and build system modifications.

Images