# GPS Tracker Memory Benchmark using ESP32

## 1. Objective

The objective of this project is to compare stack memory allocation and heap memory allocation in an ESP32-based GPS tracking application. The benchmark evaluates execution time, free heap memory, and system stability during continuous operation.

## 2. Problem Statement

Memory management is a critical aspect of embedded systems. Improper use of dynamic memory allocation may lead to memory fragmentation, increased execution time, and reduced system stability.

This project compares:

- Stack Memory Allocation

- Heap Memory Allocation

while simulating GPS data transmission over MQTT.

## 3. System Architecture

ESP32

├── GPS Generator

├── Stack Allocation Module

├── Heap Allocation Module

├── MQTT Publisher

└── Benchmark Monitor

↓

Mosquitto MQTT Broker

↓

MQTT Explorer Dashboard

## 4. Hardware Requirements

- ESP32 Development Board

- WiFi Router

- Laptop/Desktop

## 5. Software Requirements

- Arduino IDE

- ESP32 Board Package

- PubSubClient Library

- Mosquitto MQTT Broker

- MQTT Explorer

- C++

## 6. Project Components

### GPS Generator

Generates dummy GPS coordinates.

Example:

Latitude = 12.9123

Longitude = 77.6123

Speed = 45 km/h

### Stack Allocation Module

GPS data is stored using local variables.

Example:

GPSData gps;

Characteristics:

- Fast allocation

- Automatic cleanup

- No fragmentation

- Efficient memory usage

### Heap Allocation Module

GPS data is allocated dynamically.

Example:

GPSData* gps = (GPSData*)malloc(sizeof(GPSData));

Characteristics:

- Flexible memory allocation

- Requires free()

- May cause fragmentation

- Slightly slower execution

### MQTT Publisher

Publishes data to:

gps/stack

gps/heap

benchmark/stack_time

benchmark/heap_time

benchmark/freeheap

### Benchmark Monitor

Measures:

- Execution Time

- Free Heap Memory

- System Stability

Functions Used:

micros()

ESP.getFreeHeap()

## 7. Implementation

### Stack Benchmark

- Allocate GPS structure on stack.

- Generate coordinates.

- Publish through MQTT.

- Measure execution time.

### Heap Benchmark

- Allocate memory using malloc().

- Generate coordinates.

- Publish through MQTT.

- Free allocated memory.

- Measure execution time.

## 8. MQTT Dashboard

MQTT Explorer displays:

gps

├── stack

└── heap

benchmark

├── stack_time

├── heap_time

└── freeheap

This enables real-time observation of memory behavior.

## 9. Experimental Procedure

- Connect ESP32 to WiFi.

- Connect ESP32 to MQTT Broker.

- Start benchmark.

- Publish GPS packets every second.

- Run continuously for 1 hour.

- Record results.

## 10. Parameters Measured

### CPU Execution Time

Measured using:

micros()

Unit:

Microseconds (µs)

### Free Heap Memory

Measured using:

ESP.getFreeHeap()

Unit:

Bytes

### Stability

Observed for:

- Crashes

- Watchdog Resets

- Memory Leaks

- MQTT Disconnections

## 11. Results

Observed Trend:

### Stack Allocation

Advantages:

- Faster execution

- Predictable behavior

- Stable memory usage

Disadvantages:

- Limited by stack size

### Heap Allocation

Advantages:

- Flexible memory allocation

Disadvantages:

- Higher execution time

- Possible fragmentation

- Requires careful memory management

## 12. Sample Observation Table

| Parameter | Stack | Heap |
| --- | --- | --- |
| Average Execution Time | Lower | Higher |
| Memory Fragmentation | None | Possible |
| Stability | High | Moderate |
| Free Heap Variation | Minimal | Observable |
| Complexity | Low | Higher |

## 13. Advantages

- Demonstrates memory management concepts

- Useful for embedded optimization

- Provides practical benchmarking experience

- Real-time monitoring through MQTT

## 14. Applications

- IoT Devices

- GPS Trackers

- Smart Sensors

- Industrial Controllers

- Embedded Monitoring Systems

## 15. Future Enhancements

- Real GPS module integration

- SD Card data logging

- Multiple benchmark scenarios

- FreeRTOS task comparison

- Long-duration endurance testing

- Graphical performance dashboard

## 16. Conclusion

A GPS Tracker Memory Benchmark was successfully designed using ESP32 and MQTT. The project compared stack and heap memory allocation techniques under continuous GPS data generation and transmission. Results showed that stack allocation offers faster execution and greater stability, while heap allocation provides flexibility at the cost of additional overhead. The benchmark demonstrates important memory management concepts applicable to real-world embedded systems and IoT applications.