# Project 3: Heap Fragmentation Monster

## Objective

The objective of this project is to study the effects of dynamic memory allocation on the heap memory of an ESP32 microcontroller and understand how heap fragmentation occurs over time. The project demonstrates why excessive use of malloc() and free() is discouraged in embedded systems.

## Hardware and Software Used

### Hardware

- ESP32 Development Board

- USB Cable

- PC/Laptop

### Software

- Arduino IDE

- PuTTY (for serial logging)

- Python

- Pandas and Matplotlib libraries

## Theory

Embedded systems have limited memory resources. Dynamic memory allocation is performed using functions such as malloc() and free(). When memory blocks of varying sizes are allocated and released repeatedly, the heap becomes fragmented. Fragmentation results in free memory being divided into small non-contiguous blocks.

As a result, even if the total free memory appears sufficient, future allocations may fail because a large continuous block is unavailable.

This is one of the primary reasons embedded engineers avoid frequent dynamic memory allocation in long-running applications.

## Program Description

The ESP32 program repeatedly allocates memory blocks of random sizes between 50 and 500 bytes using malloc().

- If allocation succeeds, the allocated memory is initialized using memset().

- Memory allocated during even-numbered iterations is released using free().

- Memory allocated during odd-numbered iterations is intentionally left allocated.

- The free heap size is transmitted through the serial port after each allocation.

This process simulates a real-world memory usage pattern and demonstrates heap fragmentation.

## Methodology

- ESP32 code was uploaded using Arduino IDE.

- Heap statistics were transmitted through the serial port at 115200 baud.

- PuTTY was configured to automatically log serial output into a CSV file.

- The CSV file was processed using Python.

- Matplotlib was used to generate a graph of Free Heap versus Iteration.

- The graph was analyzed to observe memory usage trends.

## Results

The serial data showed a gradual reduction in available heap memory as the number of iterations increased.

A graph of Free Heap versus Iteration was generated using Python. The graph showed a downward trend indicating increasing memory consumption and fragmentation.

## Observations

- Heap memory decreased as iterations increased.

- Some memory blocks were successfully reclaimed using free().

- Memory that was not released accumulated over time.

- The heap became fragmented due to varying allocation sizes.

- Repeated allocation and deallocation can eventually cause allocation failures.

## Conclusion

The experiment successfully demonstrated heap fragmentation on the ESP32 platform. Dynamic memory allocation using malloc() and partial memory release using free() resulted in a gradual decrease in available heap memory.

The project highlights the risks associated with dynamic memory management in embedded systems and explains why embedded engineers often prefer static memory allocation techniques whenever possible.

## Learning Outcomes

- Understanding of heap memory management.

- Difference between stack memory and heap memory.

- Effects of memory leaks and fragmentation.

- Use of serial logging tools such as PuTTY.

- Data visualization using Python and Matplotlib.

- Practical understanding of memory-related issues in embedded systems.