# Live GPS Tracking Dashboard using ESP32, MQTT and Web Technologies

## 1. Objective

The objective of this project is to develop a real-time GPS tracking system using ESP32, MQTT communication protocol, Mosquitto MQTT Broker, and a web-based dashboard. The system publishes GPS coordinates from an ESP32 device and displays the location on a live map using a moving marker.

## 2. Problem Statement

Traditional GPS monitoring systems require cloud infrastructure and specialized software. The goal of this project is to create a lightweight local GPS tracking solution capable of displaying real-time location updates using open-source technologies.

## 3. System Architecture

ESP32 ↓ WiFi Network ↓ Mosquitto MQTT Broker ↓ WebSocket Server ↓ Web Dashboard (Leaflet Map) ↓ Live GPS Marker

The ESP32 generates GPS coordinates and publishes them to the MQTT topic. The Mosquitto broker receives the data and forwards it through WebSockets to the web dashboard, where the marker position is updated in real time.

## 4. Hardware Requirements

- ESP32 Development Board

- WiFi Network

- Laptop/Desktop Computer

Future Enhancement:

- NEO-6M GPS Module

- SIM7600 GSM/GPS Module

## 5. Software Requirements

- Arduino IDE

- ESP32 Board Package

- PubSubClient Library

- Mosquitto MQTT Broker

- MQTT Explorer

- Google Chrome

- HTML

- JavaScript

- Leaflet.js

- MQTT.js

## 6. Implementation

### ESP32 Side

The ESP32 performs the following operations:

- Connects to WiFi.

- Connects to MQTT Broker.

- Generates GPS coordinates.

- Publishes GPS data every second.

Example Payload:

{lat:12.9500,lon:77.6500}

Published Topic:

gps/stack

### MQTT Broker

Mosquitto Broker is configured with:

- MQTT Port: 1883

- WebSocket Port: 9001

Configuration:

listener 1883 protocol mqtt

allow_anonymous true

listener 9001 protocol websockets

### Web Dashboard

The dashboard is built using:

- OpenStreetMap

- Leaflet.js

- MQTT.js

Features:

- Real-time map display

- Automatic marker movement

- MQTT communication through WebSockets

- Browser-based visualization

## 7. Working Principle

- ESP32 publishes GPS coordinates.

- Mosquitto receives the coordinates.

- Dashboard subscribes to gps/stack topic.

- JavaScript parses received data.

- Marker position is updated.

- Map automatically centers on new coordinates.

## 8. Results

Observed Results:

- Successful MQTT communication established.

- WebSocket communication verified.

- GPS coordinates updated every second.

- Marker moved automatically on the map.

- Real-time location tracking achieved.

Example Coordinates:

12.9123, 77.6123 12.9500, 77.6500 12.9700, 77.6700

The marker successfully moved between these locations on the dashboard.

## 9. Advantages

- Low-cost implementation

- Real-time tracking

- Open-source software stack

- Easy integration with IoT devices

- Scalable architecture

## 10. Applications

- Vehicle Tracking Systems

- Fleet Management

- Asset Tracking

- Smart Logistics

- Delivery Monitoring

- Industrial IoT Systems

## 11. Future Enhancements

- Real GPS module integration

- Route history visualization

- Speed calculation

- Distance tracking

- Multiple vehicle monitoring

- Mobile application support

- Cloud storage integration

- Geofencing alerts

## 12. Conclusion

A real-time GPS Tracking Dashboard was successfully developed using ESP32, MQTT, Mosquitto Broker, WebSockets, and Leaflet.js. The system demonstrated reliable transmission and visualization of GPS coordinates through a live web interface. The project provides a strong foundation for advanced IoT-based vehicle and asset tracking applications.